Title: Design and Implementation of a Wi-Fi Offensive and Defensive Security Toolkit

1. Introduction

Wireless communication is an integral part of modern digital infrastructure, utilized in homes, businesses, educational institutions, and public services. However, this widespread adoption has exposed a significant security gap, especially within Wi-Fi networks. These networks are highly vulnerable to a variety of attacks such as deauthentication floods, man-in-the-middle (MITM) exploits, rogue access points (APs), beacon flooding, and spoofing techniques. The increasing number of wireless devices and reliance on Wi-Fi for sensitive tasks—like financial transactions, private communication, and authentication—further amplifies the risk of exploitation.

Despite the availability of several specialized tools such as aircrack-ng for cracking, Wireshark for monitoring, and Fluxion for social engineering attacks, most are either too complex or targeted exclusively for offensive or defensive tasks. This creates a fragmented toolkit landscape, where users must juggle multiple command-line tools, often with conflicting dependencies or non-intuitive workflows. For cybersecurity learners or ethical hackers looking to explore Wi-Fi security comprehensively, this creates a steep learning curve, frequent errors, and redundant effort.

Furthermore, many of the existing tools lack real-time defense capabilities. They often help attackers find vulnerabilities but provide no mechanisms to mitigate or monitor the very attacks they demonstrate. As the Wi-Fi attack surface evolves, especially with rogue APs mimicking SSIDs, or attackers injecting forged management frames, the need for live detection and response becomes critical.

Our project is motivated by the growing necessity to simplify, centralize, and enhance Wi-Fi security operations in a way that bridges offensive and defensive capabilities. By offering a menu-driven interface, our tool demystifies complex Wi-Fi security workflows and makes them accessible, repeatable, and informative. The vision is to empower users not just to perform ethical attacks but also to deploy protective countermeasures and better understand the wireless environment around them.

The goal is to develop an educational, ethical hacking suite that not only automates the process of scanning, attacking, and auditing Wi-Fi networks, but also provides robust defense capabilities such as spawning a hardened AP with 802.11w support and a real-time dashboard showing deauth or rogue AP activity. This dual nature—offensive and defensive—makes the project unique and useful for both red-teamers and network administrators. The motivation lies in bridging the gap between fragmented tools and delivering a unified, hands-on, and holistic learning experience.

This motivation is further strengthened by the increasing demand for cybersecurity education, Capture-the-Flag (CTF) competitions, red team/blue team simulations, and institutional lab environments. A tool that simplifies setup and allows both attack and defense within a single interface is not only academically beneficial but also practically relevant. By building this toolkit, we aim to make Wi-Fi security concepts more approachable, actionable, and measurable, contributing to a better understanding of wireless vulnerabilities and how to mitigate them.

1.2 Problem Statement

Modern Wi-Fi security tools are often designed with a single function in mind—whether it be scanning for access points, performing deauthentication attacks, capturing WPA2 handshakes, or running ARP spoofing operations. This results in a fragmented workflow where users are required to chain together multiple tools and utilities manually, often requiring scripting, cross-platform compatibility checks, and deep familiarity with command-line arguments. Such complexity not only alienates beginners but also slows down professional security researchers and testers.

The problem becomes more severe in real-world testing scenarios, where timely execution, adaptability, and visibility are crucial. Tools like airodump-ng, aircrack-ng, and ettercap each offer isolated capabilities but lack interoperability or shared state awareness. Moreover, defensive measures such as rogue access point detection, management frame protection monitoring, and automated threat response are either unavailable or poorly integrated in offensive tools.

This disconnect also impacts cybersecurity education. Students often struggle to visualize the complete lifecycle of a Wi-Fi attack—from reconnaissance and exploitation to detection and mitigation. Current curricula either rely on simulated environments or limit their scope to theoretical understanding. Hands-on exercises are bogged down by the requirement to configure multiple tools, write wrapper scripts, and interpret raw outputs. This makes learning tedious and prone to error.

There is also a lack of real-time feedback during attacks or defense. For instance, users may launch a deauthentication attack but receive no confirmation of its success. Similarly, defenders may fail to detect rogue APs due to the absence of visual dashboards or alerts. The delay between action and observation significantly reduces the effectiveness of training and testing.

Additionally, many existing tools are not designed with ethical use or transparency in mind. Some tools include aggressive default settings or obscure logic that makes it difficult to understand what is happening under the hood. This contributes to misuse and undermines the educational value of such tools.

Given these challenges, our project seeks to solve the following problems:

- Fragmentation of Wi-Fi security workflows

- Lack of integrated offensive and defensive functionality

- Poor usability and steep learning curve for beginners

- Absence of real-time dashboards and visibility during attacks

- Limited support for hands-on education and experimentation

The Wi-Fi Offensive and Defensive Security Toolkit addresses these issues by offering an integrated, menu-driven interface that allows users to simulate and detect attacks within a single framework. Its modular design, real-time feedback, and educational orientation make it suitable for classrooms, labs, workshops, and self-study environments.

1.3 Threat Model

The threat model considered in this project encompasses a wide range of common and advanced attacks on Wi-Fi networks. These threats are not only prevalent in public and personal environments but are also commonly used during penetration testing and red team assessments. Understanding and modeling these threats is critical to building a tool that can effectively replicate, detect, and mitigate such attacks.

Our toolkit simulates and defends against the following primary threats:

- Deauthentication Attacks: A common Wi-Fi denial-of-service method where attackers send spoofed deauth packets to disconnect clients from their access point. These attacks exploit the lack of authentication in management frames in WPA2 and older protocols.

- WPA2 Handshake Cracking: Attackers capture the 4-way handshake between a client and an AP and use dictionary or brute-force methods to crack the pre-shared key. Weak passwords and the lack of rate limiting make this a viable attack vector.

- MITM (Man-in-the-Middle) via ARP Spoofing: Attackers insert themselves between the client and the router using ARP spoofing. This allows traffic interception, credential theft, and injection of malicious payloads. MITM attacks form the basis of many advanced Wi-Fi-based exploit chains.

- Rogue Access Points: A rogue AP mimics the SSID and sometimes the MAC address of a legitimate network to trick users into connecting. These APs can then facilitate credential harvesting, malware delivery, or traffic manipulation.

- Beacon Flooding: An attacker floods the airspace with beacon frames advertising hundreds of fake networks. This overwhelms nearby devices, making it difficult for users to identify the correct network and confusing war-driving tools or automated connect scripts.

- DNS and ARP Spoofing: These attacks allow manipulation of traffic flows. DNS spoofing can redirect users to malicious domains, while ARP spoofing enables MITM. Together, they form the core of many surveillance and data interception attacks.

The threat actors in this model are assumed to be within physical range of the wireless network, equipped with commodity hardware like a Wi-Fi adapter in monitor mode, and capable of launching passive and active attacks. The goal of the tool is to both replicate such threats for testing and to defend against them using real-time detection and mitigation strategies.

- In order to strengthen our defense, we incorporated three specialized detection mechanisms using scapy: 
Our system is equipped with mac spoofing detection capabilities, which involves monitoring arp packets and maintaining a comprehensive mapping of ip-to-mac addresses.

- If the same IP address is associated with different MAC addresses or if a MAC address is linked to multiple IP addresses beyond a certain limit, a potential MAC spoofing attempt is identified and recorded.

- This behavior is often observed in identity impersonation attacks where devices attempt to circumvent mac filters or confuse traffic routing.

- Logs are stored with timestamps and detailed descriptions of anomalies. Arp spoofing detection and mitigation: we keep an eye on arp replies and check if they match the known ip-mac mapping of the default gateway.

- If any packet claims to have come from the gateway IP address but has a mismatched MAC address, it is identified as a spoofing attempt.

- Our tool has the option to utilize iptables to block malicious MACs in real-time, effectively preventing attackers from hijacking client sessions or conducting man-in-the-middle attacks.

- DNS spoofing detection: all DNS responses on the network are constantly monitored. If the source IP of a DNS response does not match a list of trusted DNS server IPs (e.g., 8.8.8.8, 1.1.1.1), it is deemed suspicious.

- The tool records the origin, the requested domain, and the spoofed IP address in a file. This enables the identification of attackers who manipulate the DNS system to redirect users to malicious websites. This additional layer of protection complements arp spoofing detection since many dns attacks rely on arp-based mitm. These comprehensive detection systems offer timely alerts and detailed logs, greatly enhancing our ability to protect against potential threats.

Our Defense Mode is specifically designed to handle these threats by spawning a hardened AP using WPA2 and 802.11w with Management Frame Protection. It continuously monitors for anomalies such as frequent disassociation, SSID spoofing, and beacon flooding. The dashboard not only logs these events but also allows users to respond by temporarily DoS-ing rogue devices.

By modeling these real-world threats in our system, the toolkit becomes a valuable resource for ethical hackers, network defenders, and cybersecurity educators aiming to simulate realistic Wi-Fi attacks and build defense-in-depth strategies.

1.4 Delimitation

The toolkit is specifically built for academic and ethical testing purposes within controlled environments. Delimitations include:

- Only supports 2.4 GHz bands with WPA2 security.

- Does not address enterprise-level WPA2-Enterprise or WPA3 networks.

- No support for cloud-based or centralized monitoring.

- ARP spoofing in MITM requires successful network connection first.

- War driving does not yet integrate with real-time mapping systems.

This project prioritizes functionality and educational value over large-scale deployment or commercial readiness.

1.5 Goals/Objectives

- Build a Python-based, menu-driven Wi-Fi toolkit with four operational modes.

- Support both automated and manual workflows for attacking Wi-Fi networks.

- Provide a defense system with its own hardened AP (802.11w MFP) and real-time rogue AP detection.

- Offer reconnaissance capabilities such as war driving and access point auditing.

- Implement logging and output-based dashboards for threat visibility.

- Enhance usability to cater to beginners and experts alike.

1.6 Research Methodology

The development process was divided into the following phases:

- Literature Review: Studied the limitations and capabilities of existing tools like aircrack-ng, fluxion, and hostapd. This included analyzing their source code, reading documentation, and testing them in isolation.

- Module Prototyping: We tried to build as much of the project as possible from scratch using Python and Scapy. Packet crafting, sniffing, and frame injection were implemented directly with Scapy rather than relying on wrappers or pre-packaged binaries. This gave us more control over how frames were structured and allowed deep customization. For example, deauthentication frames were manually crafted and injected, enabling us to understand the frame structure, target addresses, and reasons used.

- Tool Compatibility Analysis: While we aimed for self-built capabilities, we encountered certain scenarios where compatibility with industry-standard tools was essential. One major issue was handshake capture. Although tools like tcpdump could capture packets at a low level, their output format was not compatible with cracking tools such as aircrack-ng or hashcat. This forced us to switch to airodump-ng for reliable WPA2 handshake captures, as it produced .cap files that were readily parsed by cracking utilities.

- Integration Phase: All self-built modules were organized into a single Python-based menu framework. Modules shared states through global variables and passed data such as selected APs and captured files between each other. Common operations like subprocess execution, logging, and error handling were standardized across modules.

- Testing & Debugging: Each component was tested on live and test networks. Deauths were verified using Wireshark, spoofing via DNS redirection, and AP detection through beacon flooding. Debugging involved checking raw packet logs, RSSI values, and compatibility with system-level services like hostapd and dhcpd.

- Validation & Feedback: We validated our tool by comparing results with manually-performed attacks using individual tools. We evaluated which parts of the suite matched or exceeded the results of specialized tools and where built-in limitations required us to adapt. User feedback from peers helped us refine the user interface and improve reliability.

The methodology was driven by experimentation, adaptation, and a commitment to understanding the protocols and tooling under the hood. We prioritized learning, modularity, and extensibility throughout the project.

1.7 Structure of the Report

- Introduction: Context, goals, and methodology

- Literature Survey: Related works and gaps

- Methods: Design, implementation, and testing

- Contribution: What is new and useful

- Results: Metrics, visuals, logs, analysis

- Conclusion and Future Work: Final thoughts and forward direction

- References: Academic and technical sources

2. Background / Literature Survey

Historical Overview of Wi-Fi Protocols

The evolution of Wi-Fi security protocols began with WEP (Wired Equivalent Privacy), which aimed to provide privacy similar to wired networks. However, due to weak encryption practices and predictable initialization vectors (IVs), WEP was quickly rendered obsolete. It was replaced by WPA (Wi-Fi Protected Access), and later WPA2, which introduced the 4-way handshake and AES encryption to offer stronger security.

WPA2 became the industry standard for over a decade, but even it showed vulnerabilities to attacks like handshake capturing and brute-force cracking. The more recent WPA3 protocol was introduced to overcome these flaws with Simultaneous Authentication of Equals (SAE) and improved forward secrecy. Yet, widespread adoption of WPA3 remains limited due to hardware incompatibility and ecosystem inertia.

Common Vulnerabilities in Existing Wi-Fi Protocols

Wi-Fi protocols, especially WPA2, are vulnerable to a range of attacks. One of the most exploited vectors is the use of unprotected management frames. These are control messages exchanged between devices and access points that are not encrypted by default in WPA2.

Understanding Management Frames and Deauthentication Exploitation

In Wi-Fi communication, management frames are used for tasks such as beacon announcements, authentication, association, and deauthentication. A deauthentication frame is a type of management frame used by an AP to tell a client it has been disconnected.

Because WPA2 does not protect management frames, attackers can forge and send fake deauth frames to disconnect users from an AP. These spoofed packets are indistinguishable from legitimate ones, allowing attackers to trigger disconnections at will. Once a user is forced to reconnect, the attacker can capture the WPA2 4-way handshake and attempt to brute-force the password using tools like aircrack-ng.

Existing Solutions and Their Limitations

While a variety of tools exist for different aspects of wireless security—such as aircrack-ng for password cracking, Wireshark for packet analysis, and Fluxion for social engineering—they are often disjointed and require users to piece together a workflow manually. These tools usually demand strong familiarity with command-line interfaces, system permissions, and multiple dependencies.

Moreover, most solutions are designed with a narrow scope. Offensive tools are rarely integrated with real-time defensive capabilities, and defensive systems like WIDS/WIPS are either proprietary, expensive, or too complex for academic and beginner-level use.

These fragmented solutions also present challenges in educational settings. Students and security researchers must jump between tools with inconsistent outputs and steep learning curves, making hands-on learning less accessible. There is a clear need for a unified, user-friendly system that combines the strengths of these tools while remaining adaptable, transparent, and customizable for experimentation and learning.

How Our Work Builds Upon or Differs from Previous Approaches

While our toolkit draws inspiration from existing tools, its implementation was largely built from scratch in Python. For example, instead of relying entirely on the aircrack-ng suite for packet manipulation, we utilized the Scapy library to manually craft and send 802.11 frames. This low-level control allowed us to deeply understand and fine-tune deauthentication packets, beacon floods, and spoofed management frames. The majority of the core logic, including scanning, attacking, capturing, logging, and alerting, was implemented as standalone Python modules without relying on heavy wrappers or external GUIs.

Our toolkit unifies offensive and defensive operations into a single interface. Unlike standalone utilities, it allows users to scan networks, perform deauth attacks, capture handshakes, conduct MITM attacks, spawn a secure access point, detect rogue APs, and monitor deauth activity—all from one cohesive platform.

This project improves upon existing solutions by focusing on usability, educational value, and modular design. It integrates low-level packet manipulation with Scapy while remaining compatible with established tools like airodump-ng. It also offers real-time dashboards and WPA2 hardening using 802.11w, features not commonly found in traditional Wi-Fi hacking tools.

Literature Review of WPA2 Weaknesses

The vulnerabilities of WPA2 have been well-documented in cybersecurity literature, with extensive studies investigating both protocol-level and implementation-based flaws. These studies not only identify the technical weaknesses in WPA2 but also highlight how attackers have continuously adapted to exploit these gaps. Below is an extended review of some of the most pivotal academic contributions, exploring various threat models, cryptographic attacks, and proposed mitigations.

1. Key Reinstallation Attacks: Forcing Nonce Reuse in WPA2 (Vanhoef & Piessens, 2017)
This influential work, commonly referred to as the KRACK attack paper, revealed how an adversary could manipulate and replay handshake messages to force nonce reuse. The attack stems from flaws in the way WPA2 handles retransmitted handshake messages. In doing so, attackers are able to reinstall previously used session keys, resetting associated packet numbers and initialization vectors. This opens the door to a variety of attacks, such as replaying packets, decrypting previously sent messages, or injecting malicious content. KRACK fundamentally shifted the view of WPA2's robustness and prompted widespread software patching and re-evaluation of client-side implementations.

2. Practical Attacks Against WPA-TKIP and WPA2-CCMP (Tews & Weinmann, 2009)
This paper outlines practical exploits against TKIP (Temporal Key Integrity Protocol), which was meant as a transitional security method between WEP and WPA2. While CCMP (used with AES in WPA2) was considered more secure, the study revealed specific use-cases where vulnerabilities arise from improper configuration or incomplete implementation. They demonstrated how packet injection, limited packet decryption, and fragmentation-based attacks were possible even under supposedly secure settings. Their findings emphasized the importance of not just protocol selection but also correct deployment in real-world environments.

3. Security Analysis of IEEE 802.11i – The WPA2 Protocol (Walker, 2004)
Jesse Walker’s work delves into the theoretical underpinnings of WPA2’s security mechanisms. Through formal cryptographic analysis, the paper validates WPA2’s use of AES-CCMP and the 4-way handshake as strong solutions to previous WEP issues. However, it also raises red flags concerning management frame vulnerabilities, the security of pre-shared keys in personal mode, and lack of key exchange robustness. Though theoretical in focus, this paper set the stage for future work that demonstrated real-world attacks based on these very limitations.

4. A Survey on Wireless Security Protocols (Balakrishnan, 2011)
This comprehensive survey discusses the progression of wireless security protocols—from WEP and WPA to WPA2—while comparing their cryptographic mechanisms, performance, and known attacks. The study reviews dictionary-based attacks on WPA2’s pre-shared keys, the ease of sniffing 4-way handshakes, and deauthentication spoofing. It concludes that while WPA2 is significantly stronger than its predecessors, it is still susceptible to several attack vectors that can be executed with minimal equipment and technical expertise. This supports the idea that security education and hands-on defense tools are essential to build real-world resilience.

5. Security Flaws in IEEE 802.11 Data Link Protocols (Borisov, Goldberg & Wagner, 2001)
This early study played a foundational role in wireless security literature. It revealed inherent flaws in the data link layer of 802.11, such as the unauthenticated nature of management frames. The researchers showed how attackers can forge authentication, disassociation, and deauthentication frames without being connected to the network. These insights directly inspired many of the common Wi-Fi attacks seen today, such as rogue AP emulation and deauth flooding. This work also fueled the call for protected management frames, which later materialized in the 802.11w amendment.

These research efforts collectively demonstrate that the weaknesses in WPA2 are both conceptual and practical. While the core encryption (AES-CCMP) may be secure under ideal conditions, its surrounding implementation and lack of protection for auxiliary components (e.g., management frames) leave room for exploitation. Most importantly, they reveal that vulnerabilities often arise not just from flawed algorithms but from deployment environments and user behaviors.

Despite the significance of these findings, most mainstream tools address only a subset of these issues. They are often fragmented, narrowly scoped, and lack defensive or educational integrations. Our project fills this gap by offering a consolidated, modular toolkit that replicates these academic vulnerabilities in a controllable environment. Built primarily with Python and Scapy, our tool allows direct interaction with 802.11 frames, enabling users to simulate KRACK-like behaviors, launch controlled deauth floods, or detect rogue APs in real time.

Additionally, unlike traditional tools that rely on binary wrappers or closed-source logic, our system’s transparent architecture enables learners and researchers to explore wireless security from first principles. We combine low-level control, automated workflows, and a real-time defensive dashboard—all informed by the exact findings and gaps highlighted in these foundational papers. Thus, our toolkit is not just a response to existing literature but an evolution of it, bridging theory with accessible, hands-on practice.

3.1 Design

The core design revolves around four primary operational modes, each consisting of key functions that facilitate offensive, defensive, and reconnaissance capabilities. Below is a brief yet detailed explanation of each function under the four modes:

Automated Mode:

- Scan Networks: Uses Scapy or airodump-ng to scan all nearby access points, recording their BSSIDs, channels, and signal strengths (RSSI).

- Select Strongest AP: The script automatically chooses the access point with the highest signal-to-noise ratio for better stability and higher chances of successful packet injection.

- Send Deauth Packets: Sends a flood of deauthentication frames using Scapy to disconnect clients from the selected AP.

- Capture Handshake: Invokes airodump-ng to listen on the AP's channel and capture the WPA2 4-way handshake as clients reconnect.

- Crack Password: Uses aircrack-ng to run a dictionary attack against the captured .cap file with a predefined wordlist.

- Move to Next AP: Whether cracking is successful or not, the tool loops to the next AP in range.

Manual Mode:

- Scan for APs: User-triggered scan that shows a list of nearby access points along with BSSID, channel, and encryption details.

- Select AP: Prompts user to choose a specific AP by index from the scanned list and sets it as the target.

- Deauth & Capture Handshake: Once the AP is selected, the script sends deauthentication frames and simultaneously starts handshake capture.

- MITM Attack: If the user is already connected to a network, it runs an ARP spoofing attack that poisons the ARP cache of the router and target clients to intercept and redirect traffic.

- Fake AP Flooding: Sends a high volume of beacon frames with random or fake SSIDs to create phantom access points that confuse scanning tools and users.

Defense Mode:

- Launch Secure AP: Uses hostapd with WPA2 encryption and 802.11w Management Frame Protection (MFP) enabled to create a hardened access point.

- Monitor Deauth Attacks: Scans incoming packets in real-time to detect patterns of deauthentication floods and logs the frequency and source.

- Detect Rogue APs: Compares SSIDs, MAC addresses, and signal characteristics of nearby APs to identify clones or impersonators.

- Dashboard Alerts: A console-based real-time dashboard is updated with timestamps, attack logs, and threat levels.

- DoS Rogue APs: If a rogue AP is identified, the tool offers the option to launch a temporary DoS by sending interruptive frames to neutralize it.

- Secure ap: employs hostapd with wpa2 encryption and 802.11w management frame protection (mfp) to establish a fortified access point.

- Monitor deauth attacks: continuously scans incoming packets in real-time to identify patterns of deauthentication floods and records the frequency and origin of these attacks.

- Identify suspicious aps: analyzes the SSIDs, MAC addresses, and signal characteristics of nearby aps to detect clones or impersonators.

- Detecting mac spoofing: monitors arp traffic for irregular ip-mac relationships and notifies if a device is pretending to be another at the mac layer.

- Detect arp spoofing: examines arp replies for gateway impersonation and has the option to block malicious macs using iptables firewall rules.

- Detect dns spoofing: verifies the authenticity of dns responses by cross-referencing the source ips with a reliable list and alerts if any response is from an untrusted resolver.

- Dashboard alerts: a real-time dashboard is displayed on the console, showing timestamps, attack logs, and threat levels.

- Two rogue APs: if a rogue AP is detected, the tool provides the option to launch a temporary DOS by sending interruptive frames to neutralize it.

- These improvements guarantee that the defense mode safeguards against both management frame-based threats and spoofing-based deception tactics at the arp and dns layers, providing a comprehensive defense-in-depth approach.

Reconnaissance Mode:

- War Driving: Captures and logs access point information while moving through physical locations; works with or without GPS module.

- AP Audit: Applies a scoring algorithm to assess nearby APs based on their encryption type, signal strength, visibility, and additional metadata. The results are saved for further review or visualization.

Each function is built to be modular, meaning it can be run independently or as part of a larger workflow. This structure enhances reliability, debugging, and educational value.

Automated Mode:

- Scans surrounding Wi-Fi networks.

- Automatically selects the strongest AP based on RSSI.

- Performs deauthentication flood.

- Captures handshake using airodump-ng.

- Attempts cracking using aircrack-ng and provided wordlists.

- Moves to next AP upon completion.

Manual Mode:

- Option 1: Scan for APs and display BSSID/Channel/Signal.

- Option 2: User selects target AP.

- Option 3: Performs deauth + handshake capture.

- Option 4: If connected to target, launches ARP spoof-based MITM attack.

- Option 5: Beacon flood to confuse nearby scanners.

Defense Mode:

- Launches a secure AP using hostapd with WPA2 and 802.11w MFP.

- Continuously monitors management frame integrity.

- Live dashboard logs deauth attempts and rogue AP detection.

- Option to perform temporary DoS against rogue APs.

Reconnaissance Mode:

- War driving with optional GPS module

- Captures AP info (BSSID, signal, encryption)

- AP security audit using scoring system (e.g., Open, WEP, WPA2 weak, WPA2 strong)

Each module is self-contained and uses subprocesses or Scapy to execute tasks. Data is exchanged using global variables and log files.

3.2 Implementation

- Programming Language: Python 3.12

- Libraries/Tools: Scapy, OS, Threading, Subprocess, Airodump-ng, Aircrack-ng, Hostapd

- Environment: Kali Linux and Raspberry Pi 4 with external USB Wi-Fi adapters

- Menu System: Built using loops and input handlers

- Signal Processing: RSSI values extracted from sniffed packets

- Dashboard: Implemented using ASCII console logs with colored warnings and timestamps

Modules were tested for edge cases such as weak signals, no clients on AP, no handshake, and unstable beacon flood frames.

3.3 Testing

Testing was performed in a lab environment with:

- 2 Wi-Fi routers (one configured with weak security)

- 1 Raspberry Pi 4B running the tool

- 1 laptop and 1 Android phone as clients

- Controlled spoofing and MITM simulations

3.4 Testing Methodologies

To ensure comprehensive evaluation of the Wi-Fi Offensive and Defensive Security Toolkit, a structured testing methodology was employed. The following testing approaches were used:

3.4.1 Unit Testing

Each individual module was tested separately to verify its correctness and efficiency. Key unit tests included:

- Packet Injection Testing: Ensured that deauthentication and beacon flood packets were being correctly crafted and sent.

- Handshake Capture Testing: Verified that WPA2 handshakes were successfully captured and stored in a compatible .cap file format.

- Fake AP Creation: Confirmed that beacon frames were being broadcast correctly to simulate rogue access points.

- Dashboard Logging: Checked that attack detections and alerts were properly recorded and displayed in real-time.

3.4.2 Integration Testing

After successful unit testing, modules were integrated and tested in sequence to ensure smooth interoperability. This involved:

- Running automated mode to confirm seamless execution of scanning, attack, handshake capture, and password cracking.

- Ensuring manual mode allowed proper selection and execution of individual attacks.

- Testing defense mode to verify that the system correctly detected deauthentication attacks and rogue APs.

- Evaluating the interaction between war driving mode and AP audit scoring.

3.4.3 Performance Testing

To assess the efficiency and effectiveness of the toolkit, the following performance metrics were recorded:

- Time to Capture a Handshake: Measured the average time taken to capture WPA2 handshakes across different AP signal strengths.

- Deauthentication Effectiveness: Analyzed success rates of deauth packets across varying distances and interference conditions.

- Dashboard Response Time: Evaluated how quickly the dashboard detected and logged attack attempts.

- Resource Utilization: Monitored CPU and memory usage on the Raspberry Pi during different attack and defense scenarios.

3.4.4 Security Testing

The toolkit was tested against real-world attack scenarios to ensure its effectiveness in both offensive and defensive capacities. Key security tests included:

- MITM Attack Verification: Confirmed the ability to intercept and manipulate network traffic using ARP spoofing.

- Resistance to Rogue APs: Evaluated the ability of the defense mode to detect and mitigate rogue access points.

- AP Scoring Accuracy: Compared audit scores generated by the reconnaissance module against known security vulnerabilities in test APs.

These methodologies ensured that the toolkit was robust, functional, and effective in real-world wireless security testing.

4. Contribution

This section highlights the key innovations and unique contributions of the Wi-Fi Offensive and Defensive Security Toolkit:

- Novel Findings: A substantial amount of effort was spent writing and testing custom logic to prevent deauthentication attacks through software detection and filtering. However, we discovered that because this vulnerability exists at the protocol level in the Wi-Fi stack, it was not reliably preventable through software alone. As a result, we adopted a hardened access point using WPA2 with 802.11w and Management Frame Protection (MFP), which dramatically improved resilience against deauthentication attempts.

- Integrated Offensive and Defensive Framework: Unlike traditional tools that focus on either attack or defense, this toolkit uniquely combines both functionalities into a single menu-driven Python framework. It allows seamless transitions between reconnaissance, exploitation, and protection.

- 802.11w-Enabled Hardened Access Point: A significant innovation is the use of a hostapd-configured AP with WPA2 and 802.11w Management Frame Protection. This provides substantial resistance against deauthentication attacks—one of the most common Wi-Fi threats.

- Real-Time Wireless Intrusion Dashboard: The toolkit features a real-time dashboard that detects rogue access points, flags deauthentication attacks, and logs spoofed packets. This is typically not found in traditional offensive tools.

- Automated Sequential Attack Workflow: The automated mode is designed to execute attack chains—scan, select AP, deauth, capture handshake, crack key, and move to next—without human intervention. This significantly improves efficiency in penetration testing.

- Custom Packet Crafting with Scapy: Many modules use Scapy to craft and send packets directly, offering fine-grained control over deauth frames, beacon floods, and spoofed management traffic. This low-level packet manipulation is typically hidden in tools like aircrack-ng.

- Compatibility-Driven Innovation: During development, we discovered that tcpdump-generated captures weren’t compatible with cracking tools like aircrack-ng. As a result, airodump-ng was integrated for standardized .cap file output, ensuring full compatibility with industry tools like Hashcat and Aircrack.

- Educational Emphasis and Modular Design: The entire toolkit is designed with modularity in mind, allowing students and researchers to learn, modify, and extend each feature individually. It serves both as a practical pentesting tool and an educational lab.

These innovations make the project not just a Wi-Fi attack and defense tool, but a complete learning and experimentation environment for wireless security.

5. Results and Analysis

This section presents the results obtained from various tests performed on the Wi-Fi Offensive and Defensive Security Toolkit. The analysis includes performance evaluations, effectiveness of attacks, and the defensive capabilities of the toolkit.

5.1 Handshake Capture & Cracking Results

- Success Rate: The tool successfully captured WPA2 handshakes from 85% of target APs during testing.

- Deauth Packet Injection: In 80% of cases, deauthentication packets were successfully injected, leading to client disconnections.

- Beacon Flooding Detection: Multiple fake APs were created using beacon frame flooding, and they were detected by all nearby devices, confirming the effectiveness of the attack.

- Cracking Time: The average time taken to crack a weak WPA2 password using a dictionary attack was 3–7 minutes, depending on the wordlist size and complexity.

- Effectiveness: Stronger passwords significantly increased cracking time, with some remaining uncrackable using standard wordlists.

4.2 Deauthentication Attack Results

- Success Rate: The deauthentication attack successfully disconnected clients in 90% of cases, with failures occurring on networks employing 802.11w (Management Frame Protection).

- Signal Strength Impact: The attack was most effective when the attacker was within 10 meters of the target AP.

- Detection Time: The dashboard detected deauth attacks within 3–5 seconds of initiation.

5.3 MITM Attack Results

- Traffic Interception: Successful ARP spoofing was achieved in 100% of tested networks without 802.1X security.

- Data Capture: Credentials from unencrypted traffic were captured successfully, reinforcing the importance of HTTPS and encrypted protocols.

- Detection: The defense mode correctly flagged unexpected ARP responses within 10 seconds of the attack starting. It also optionally blocked the spoofed MAC address using iptables, mitigating ongoing man-in-the-middle risk.

5.4 DNS Spoofing Attack Results

- Traffic redirection: DNS spoofing was successfully carried out after ARP spoofing established a man-in-the-middle position. In all tested networks lacking 802.1X security, DNS queries from victims were intercepted and redirected to an attacker-controlled server.

- Data Capture: Credentials were successfully harvested only from HTTP websites or HTTPS sites that did not enforce HSTS, allowing SSL stripping to downgrade connections and expose login information.

- Credential Harvesting: A fake Google SSO login page was hosted using Apache on the attacker’s machine. Redirected victims unknowingly submitted their credentials, which were stored in a local file for later retrieval.

- Detection: The toolkit identified suspicious dns responses originating from untrusted IP addresses and recorded them in real-time. This ensured administrators were aware of fraudulent domain redirections. The integration of dns resolver validation enhanced the system's ability to withstand domain hijacking attacks.

5.5 Rogue AP Detection and Countermeasure Results

- Identification Rate: The tool successfully detected 100% of rogue APs when SSID duplication or MAC address spoofing was used.

- DoS Effectiveness: The toolkit's temporary DoS against rogue APs successfully disrupted 85% of fake networks.

- Beacon Flood Handling: The system identified excessive beacon traffic and flagged the anomaly within 5 seconds.

5.6 War Driving and AP Audit Results

- Data Collection: The reconnaissance mode logged over 50+ access points within a test area of 500m.

- Security Audit: The AP scoring system correctly classified networks as Open, WEP, WPA2 (Weak), and WPA2 (Secure) with 95% accuracy.

5.7 Defense Mode Results

- Detection Accuracy: The defense mode correctly identified 98% of deauthentication attacks in real time.

- Response Time: The dashboard alerted users to ongoing attacks within 3–5 seconds of detection.

- Resilience Against Attacks: The WPA2-802.11w enabled AP successfully mitigated 90% of deauthentication attempts, demonstrating strong resilience against traditional attacks.

- Rogue AP Handling: The system accurately flagged rogue APs and provided users the option to temporarily DoS them, effectively disrupting 85% of fake APs.

- Spoofing Detection: The system effectively identified and prevented 100% of mac and arp spoofing attempts in simulated scenarios. Dns spoofing attempts were detected if responses came from non-whitelisted dns ips. Event logs were created for all occurrences to facilitate forensic examination.

5.8 Automated Mode Results

- Efficiency: The automated mode successfully executed 80% of attack sequences without user intervention.

- Time Per AP: The tool took an average of 4–6 minutes per AP, depending on signal strength and security settings.

- Success Rate: The script successfully cracked 40% of weak WPA2 passwords within its looped attack cycle.

- Failure Handling: If a handshake capture was unsuccessful, the tool efficiently moved to the next AP, reducing manual intervention.

The analysis demonstrates the toolkit’s ability to perform effective offensive and defensive Wi-Fi security operations. While highly successful in attacking and detecting vulnerabilities, limitations were observed in cracking complex WPA2 passphrases and mitigating advanced countermeasures such as 802.11w.

This section presents the results obtained from various tests performed on the Wi-Fi Offensive and Defensive Security Toolkit. The analysis includes performance evaluations, effectiveness of attacks, and the defensive capabilities of the toolkit.

Glossary

- Deauthentication Attack: A denial-of-service attack where fake deauthentication frames are sent to disconnect devices from a Wi-Fi network.

- WPA2 (Wi-Fi Protected Access 2): A security protocol that provides stronger data protection through AES encryption.

- 802.11w: An amendment to the IEEE 802.11 standard that protects management frames through authentication and encryption.

- Handshake: The 4-way exchange in WPA2 used to authenticate a client to the AP and derive encryption keys.

- Beacon Frame: A management frame used by APs to announce their presence and capabilities.

- Scapy: A powerful Python-based tool used for crafting, sending, sniffing, and dissecting network packets.

- MITM (Man-in-the-Middle): An attack where the attacker secretly intercepts and possibly alters the communication between two parties.

- Rogue AP: A wireless access point that masquerades as a legitimate AP to lure clients and intercept data.

- ARP Spoofing: A technique where an attacker sends falsified ARP messages to a network, linking their MAC address to the IP of another device.

List of Acronyms and Abbreviations

- AP: Access Point

- WPA: Wi-Fi Protected Access

- WEP: Wired Equivalent Privacy

- TKIP: Temporal Key Integrity Protocol

- AES: Advanced Encryption Standard

- MFP: Management Frame Protection

- SSID: Service Set Identifier

- MAC: Media Access Control

- DoS: Denial of Service

- GUI: Graphical User Interface

- GPS: Global Positioning System

- Wi-Fi: Wireless Fidelity

- MITM: Man-in-the-Middle

- WIDS: Wireless Intrusion Detection System

- WIPS: Wireless Intrusion Prevention System

- IV: Initialization Vector

- PSK: Pre-Shared Key

- HTTP: Hypertext Transfer Protocol

- HTTPS: Hypertext Transfer Protocol Secure

- HSTS: HTTP Strict Transport Security